PROJECT:  BUILDING A TYPICAL SECURITY ARCHITECTURE

Author: Sampath Ajit Chittella(2025)

OBJECTIVE:

The purpose of this laboratory is to introduce students to different network security devices by having access to the network device and configuring the network devices IP addresses. Configuring the IP addresses of the network devices will expose students to some basic command lines and serve as a start to their introduction to different network security devices.

NOTES:

- Use the network topology in Fig 3 in page 3 to connect the devices, configure and assign IP addresses to interfaces of the network devices as provided along this guide.

- Use Fig. 1 options for the Palo Alto firewall image:

RAM (MB): 8192

Console: telnet

Fig.1

- Add public network to eve-ng (Fig. 2)

- Right-click on an empty part of the eve-ng window

- Select network.

- Change the Name/Prefix to NAT, select the globe1.png icon, and select NAT for the type as shown in the image below:

Fig.2

- Add private network to eve-ng (Fig.2a)

- Follow steps i. and ii., just above.

- Change the Name/Prefix to Private, select the cloud.png icon, and select private for the type as shown in the image below:

Fig.2a

TOPOLOGY

Fig. 3

TASK 1: EIU MAIN CAMPUS BASIC ROUTER CONFIGURATION

-On the EIU Main Campus Gateway Router

Router> enable

Router# configure terminal

Router(config)# hostname Campus-Gateway

Campus-Gateway(config)# int g0/0

Campus-Gateway(config-if)# ip address dhcp

Campus-Gateway(config-if)# no shutdown

Campus-Gateway(config)# exit

Campus-Gateway(config)# int g0/1

Campus-Gateway(config-if)# ip address 90.0.1.2 255.255.255.248

Campus-Gateway(config-if)# no shutdown

Campus-Gateway(config-if)# exit

Campus-Gateway(config)# int g0/2

Campus-Gateway(config-if)# ip address 192.168.10.100 255.255.255.0

Campus-Gateway(config-if)# no shut

Campus-Gateway(config-if)# exit

Campus-Gateway(config)# exit

Campus-Gateway# copy run start (Press Enter twice)

INSTRUCTION: Use “show run” and “show ip interface brief” to re-check the parameters inputted in this task.

TASK 2: PALO ALTO INITIAL WEB ADMINISTRATION AND MANAGEMENT

(NOTE: The following steps are applicable to both Palo Alto Firewalls at both sites)

- Set up a PC for the Configuration of the Two Palo Alto firewalls

Select a Windows PC and connect it to the private network.

Fig. 4

- Set up the Palo Alto firewall

Start the Palo Alto firewall and wait for a while for its booting process to complete

(NOTE: typical booting process of this Palo Alto image takes about 25 minutes, you can keep connecting other parts of the network in the meantime).

NOTE: The Firewall logs in after several tries (even if we use the correct credentials), so if you’re not able to access, please wait a few minutes before you try again.

When the booting is completed, use username/password of admin/admin to access the CLI of the Palo Alto firewall, and use the following commands to assign its IP address:

admin@PA-VM> configure mode

admin@PA-VM# set deviceconfig system type static

admin@PA-VM# set deviceconfig system ip-address 192.168.1.x netmask 255.255.255.0

admin@PA-VM# commit

(NOTE: Use ip address in the range 192.168.1.1 to 192.168.1.254 for the Paloalto Firewalls, but do not use the same ip address for any two devices connected to the private network)

- Configure the Windows PC IP address

You can assign any IP address ranging from 192.168.1.1 to 192.168.1.254/24. Do not assign an IP address that is already in use by any of the Palo Alto Firewalls.

To verify the connection between the PC and the Palo Alto firewalls, open each Palo Alto firewall and ping the IP address as follows:

admin@PA-VM> ping host 192.168.1.x

(NOTE: Use ctrl+c to stop the reply packets)

Click on the Click on Google Chrome browser icon and type https://192.168.1.x on the browser tab to open a Palo Alto configuration home page. (NOTE: Select the Advanced… then Accept the Risk and Continue)

Use username/password of admin/Tech@lab to login to the Palo Alto GUI.

Fig.5

Open a new tab on the browser and type ‘https://192.168.1.x’, the ip address of the second Paloalto Firewall to access its home page.

- Create a role for an assistant administrator which will allow access to all firewall functionality through the WEBUI except Monitor, Network, Privacy, and Device. Create an account using this role.

- Click Device → Admin Roles. (in the left pane)

- Click Add in the lower left of the panel and create a new admin role:

- Type Policy Admins in the name window

| Web UI tab categories | Click the following major categories to disable them: - Monitor - Network - Device - Privacy The remaining major categories should remain enabled. |
| --- | --- |

- Click OK to continue.

TASK 3: CREATE A GENERAL ACCOUNT

- Click Device → Administrators.

- Click Add in the lower left corner of the panel. Configure a new administrator account:

| Name field | Type ip-admin |
| --- | --- |
| Password/Confirm | Type paloalto |
| Administrator type | Select Role Based |
| Profile | Select Policy Admins (scroll down menu) |
| SSH & pass profile | Leave it unchecked |

- Click OK

- To save your configuration, click the Commit link at the top‐right of the WEBUI. (Close to Device tab)

- Click OK to the commit pop- up window and wait until the commit process completes, then click Close.

IMPORTANT NOTE: Every time you make a change in a PALO ALTO firewall, if you do not “Commit” the change will not take effect. Please beware of this fact throughout this guide, even if it is not mentioned in every change.

The web page will appear as shown below (Fig. 6):

Fig. 6

TASK 4: CONFIGURE THE MAIN CAMPUS PALO ALTO INTERFACES

- Click on Network tab.

- From the left-hand pane, configure the management interface.

- Click on Interface Mgmt.

- Click on Add button (At the bottom left of the dialogue box).

- Name: Ping-Only

- Check Ping box.

- Click OK.

- Click on Interfaces at the left-hand pane. Click on Ethernet1/1 to configure it.

- Change Interface Type to Layer3 using the drop-down menu.

- Click on IPV4 tab, click on Add on the opened window (ensure the Static type category is selected).

- Enter the IP address 90.0.1.1/29. Click any open space.

- Click Advanced tab, select “ping-only” for management profile, then click OK, OK.

- Configure Ethernet1/2 with IP address 10.144.1.2/29 following the steps for Ethernet1/1 above.

- Configure Ethernet1/3 with IP address 172.16.1.1/24 following the steps for Ethernet1/1 above.

- Click on Zones, click Add, use the following parameters:

- Name: Trust

- Type: Layer3

- Interface: Click Add, select ethernet1/2

- Click OK

- Add the Untrust Zone with the following details:

- Name: Untrust

- Type: Layer3

- Interface: Click Add, select ethernet1/1

- Click OK

- Add the DMZ Zone with the following details:

- Name: DMZ

- Type: Layer3

- Interface: Click Add, select ethernet1/3

- Click OK

- Click on The NETWORK tab, then on Virtual Routers

- Click on default.

- Click Add, and then add the three Palo Alto interfaces Ethernet1/1, Ethernet1 /2 and Ethernet 1/3 and click OK when done.

INSTRUCTION: ** Please commit your configurations before proceeding.

You should see figures 7,8, and 9 below at this point, if you don’t, please call the attention of your instructor.

Fig. 7

Fig. 8

Fig. 9

FIGURE 3 REPRODUCED FOR YOUR CONVENIENCE

TASK 5:

CONFIGURE THE CISCO ASA

Ciscoasa>

Ciscoasa> enable (Press Enter twice)

Ciscoasa# configure terminal

Ciscoasa(config)# hostname ASA-FW-IPS

ASA-FW-IPS(config)#

ASA-FW-IPS(config)# int gig0/1

ASA-FW-IPS(config-if)# nameif Outside

ASA-FW-IPS(config-if)# ip address 10.144.1.1 255.255.255.248

ASA-FW-IPS(config-if)# no shutdown

ASA-FW-IPS(config-if)# exit

ASA-FW-IPS(config)# int gig0/3

ASA-FW-IPS(config-if)# nameif Inside

ASA-FW-IPS(config-if)# ip address 172.16.32.2 255.255.255.248

ASA-FW-IPS(config-if)# no shutdown

ASA-FW-IPS(config-if)# exit

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

TASK 6: CONFIGURE THE INTERNAL ROUTER

Internal-Router> enable

Internal-Router# conf t

Internal-Router(config)# int g0/0

Internal-Router(config-if)# ip address 172.16.32.1 255.255.255.248

Internal-Router(config-if)# no shutdown

Internal-Router(config-if)# exit

Internal-Router(config)# int g0/1

Internal-Router(config-if)# ip address 10.234.100.1 255.255.255.0

Internal-Router(config-if)# no shutdown

Internal-Router(config-if)# end

Internal-Route# copy run start (Press Enter twice)

INSTRUCTION: Use “show run” and “show ip interface brief” to re-check the parameters inputted in this task.

TEST:

At this point, Check some basic connectivity between the devices with ping command:

- Log on to Internal-Router, ping 172.16.32.2 (ASA G0/3 interface)

- Log on to ASA-FW-IPS, ping 172.16.32.1 (Internal-Router G0/0 interface)

- Log on to ASA-FW-IPS, ping 10.144.1.1 (Palo Alto eth1/2 interface)

- All these pings should be successful, otherwise call the attention of your Instructor/GA

- Check that the E1/2 Palo Alto port is now green (Provided the Palo Alto firewall at EIU main campus is configured already)

TASK 7: CONFIGURE THE CHAMPAGNE REMOTE SITE  Champagne_GatewayRouter> enable

Champagne_GatewayRouter# conf t

Champagne_GatewayRouter(config)# int g0/0

Champagne_GatewayRouter(config-if)# ip address dhcp Champagne_GatewayRouter(config-if)# no shutdown Champagne_GatewayRouter(config-if)# exit

Champagne_GatewayRouter(config)# int g0/1

Champagne_GatewayRouter(config-if)# ip address 80.0.1.2 255.255.255.248

Champagne_GatewayRouter(config-if)# no shutdown Champagne_GatewayRouter(config-if)# exit

Champagne_GatewayRouter(config)# int g0/2

Champagne_GatewayRouter(config-if)# ip address 192.168.10.101 255.255.255.0

Champagne_GatewayRouter(config-if)# no shutdown

Champagne_GatewayRouter(config-if)# exit

Champagne_GatewayRouter(config)# exit

Champagne_GatewayRouter#

INSTRUCTION: Use “show run” and “show ip interface brief” to re-check the parameters inputted in this task.

TASK 8: CONFIGURE CHAMPAGNE  REMOTE SITE PALO ALTO  FIREWALL

Follow the same procedures above for EIU Main Campus Palo Alto to configure champagne Palo alto using the details on the topology. Note the IP address and the zones of each of the interface to configure below:

- Ethernet1/1 ➔ 80.0.1.1/29

- Ethernet1/2 ➔ 10.234.200.1/24

- Zones: Untrust ➔ ethernet1/1

- Zones: Trust ➔ ethernet1/2

- Interface Mgmt: Ping-Only, check Ping box.

- Default virtual routers: add e1/1 and e1/2.

INSTRUCTION: ** Please commit your configurations before proceeding.

Check if your Champagne Palo Alto looks like the following figures:

Fig. 10

Fig. 11

TEST:

At this point, check some basic connectivity between the devices with ping command:

- Log on to Champagne_GatewayRouter and ping 80.0.1.1 (Palo Alto eth1/1 interface)

This ping should be successful, otherwise call the attention of your Instructor/GA

TASK 9: CONFIGURE DHCP SERVERS ON BOTH SITES

For EIU Main Campus Site:

- Log on to Internal Router and configure DHCP Server as follows: Internal_Router(config)# ip dhcp excluded-address 10.234.100.1 Internal_Router(config)# ip dhcp pool CAMPUSPOOL

Internal_Router(dhcp-config)# network 10.234.100.0 /24

Internal_Router(dhcp-config)# dns-server 8.8.8.8 4.2.2.2

Internal_Router(dhcp-config)# default-router 10.234.100.1

INSTRUCTION: Use “show run” and “show ip interface brief” to re-check the parameters inputted in this task.

- Connect the LAN Hosts as shown on the topology. Set the IP configuration to dynamic:

- For the Virtual PC (VPCS), type “ip dhcp” command

Type “show ip” command to check the IP address obtained from the dhcp server.

- For the Windows PC, take the following steps:

Start Menu>Control Panel>Network and Sharing center>Change adapter settings> click on the LAN that shows up >>>  >>>Properties > Internet Protocol (TCP/IPv4) > Properties

Fig. 12

Click on “Obtain an IP address automatically” and “Obtain DNS server address automatically” to dynamically assign IP address to the PC, then Click OK once you are done.

Open the command prompt and type the command “ipconfig /all” to check the IP address.

Inform the GA if the hosts did not receive IP address.

- From the Hosts, ping the default gateway IP address. This must be successful, otherwise report to the GA.

For the Champagne Remote Site:

Log on to the Palo Alto firewall.

- Click on Network tab.

- On the left-hand pane, click on DHCP.

- On the DHCP Server window, click Add, on the Interface option, click on the drop-down menu, select ethernet1/2.

- Click on the Timeout radio button and leave it to 1 day.

- Under the options tab:

  - For Gateway, enter 10.234.200.1

  - For Primary DNS, enter 8.8.8.8

  - For Secondary DNS, enter 4.2.2.2

- Under the Lease tab, for IP Pool section, click Add and enter 10.234.200.2- 10.234.200.100

INSTRUCTION: ** Please commit your configurations before proceeding.

TESTING CONNECTIVITIES

From all the LAN Hosts, ping the default gateways – this should be successful. IMPORTANT: DISABLE ALL OTHER INTERFACES (Except the one used), and the Windows firewall defender of the Windows PCs. Set the PCs to obtain IP address and DNS configurations automatically. But note that the LAN Hosts have no internet connection as they cannot go online. Test if the PCs can go online from both sites.

TASK 10: VPN CONFIGURATION on MAIN CAMPUS ROUTER

The aim of this task is to connect the two sites over the public network. As of now, both sites cannot communicate LAN-TO-LAN. This is expected because the two sites are still isolated.

Configure the VPN on the Campus Gateway Router

Create tunnel interface.

Campus-Gateway(config)# interface tunnel 12

Campus-Gateway(config-if)# ip add 10.0.0.1 255.255.255.0

Campus-Gateway(config-if)# tunnel source g0/2

Campus-Gateway(config-if)# tunnel destination 192.168.10.101

VPN Phase I

Campus-Gateway(config)# crypto isakmp policy 10

Campus-Gateway(config-isakmp)# auth pre-share

Campus-Gateway(config-isakmp)# encryption aes

Campus-Gateway(config-isakmp)# hash sha

Campus-Gateway(config-isakmp)# group 5

Campus_Gateway(config-isakmp)# exit

Campus-Gateway(config)# crypto isakmp key CapstonE#1! address 192.168.10.101

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

VPN Phase II

Campus-Gateway(config)# crypto ipsec transform-set EIUVPN esp-aes esp-sha-hmac

Campus-Gateway(cfg-crypto-trans)# exit

Create the ipsec profile to link Phase II

Campus-Gateway(config)# crypto ipsec profile VPNPROFILE

Campus-Gateway(ipsec-profile)# set transform-set EIUVPN

Campus-Gateway(ipsec-profile)# exit

Apply the profile to the appropriate interface.

Campus-Gateway(config)# int tunnel 12

Campus-Gateway(config-if)# tunnel mode ipsec ipv4

Campus-Gateway(config-if)# tunnel protection ipsec profile VPNPROFILE

Campus-Gateway(config-if)# exit

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

TASK 11: RUN OSPF ON THE MAIN CAMPUS ROUTER

Campus-Gateway(config)# router ospf 1

Campus-Gateway(config-router)# network 10.0.0.0 0.0.0.255 area 0

Campus-Gateway(config-router)# network 192.168.10.0 0.0.0.255 area 0

Campus-Gateway(config-router)# network 90.0.1.0 0.0.0.7 area 0.0.0.1

Campus-Gateway(config-router)# default-information originate

Campus-Gateway(config-router)# exit

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

TASK 12: VPN CONFIGURATION ON THE CHAMPAGNE REMOTE SITE ROUTER

Create tunnel interface

Champagne_GatewayRouter(config)# interface tunnel 21

Champagne_GatewayRouter(config-if)# ip add 10.0.0.2 255.255.255.0 Champagne_GatewayRouter(config-if)# tunnel source g0/2

Champagne_GatewayRouter(config-if)# tunnel destination 192.168.10.100

VPN Phase I

Champagne_GatewayRouter(config)# crypto isakmp policy 10 Champagne_GatewayRouter(config-isakmp)# auth pre-share Champagne_GatewayRouter(config-isakmp)# encryption aes Champagne_GatewayRouter(config-isakmp)# hash sha

Champagne_GatewayRouter(config-isakmp)# group 5

Champagne_GatewayRouter(config-isakmp)# exit

Champagne_GatewayRouter(config)# crypto isakmp key CapstonE#1! address 192.168.10.100

VPN Phase II

Champagne_GatewayRouter(config)# crypto ipsec transform-set EIUVPN esp-aes esp-sha-hmac

Champagne_GatewayRouter(cfg-crypto-trans)# exit

Create the ipsec profile to link Phase II

Champagne_GatewayRouter(config)# crypto ipsec profile VPNPROFILE Champagne_GatewayRouter(ipsec-profile)# set transform-set EIUVPN Champagne_GatewayRouter(ipsec-profile)# exit

Apply the profile to the appropriate interface.

Champagne_GatewayRouter(config)# int tunnel 21

Champagne_GatewayRouter(config-if)# tunnel mode ipsec ipv4

Champagne_GatewayRouter(config-if)# tunnel protection ipsec profile VPNPROFILE

Champagne_GatewayRouter(config-if)# exit

Run OSPF on the Router

Champagne_GatewayRouter(config)# router ospf 1

Champagne_GatewayRouter(config-router)# network 10.0.0.0 0.0.0.255 area 0

Champagne_GatewayRouter(config-router)# network 192.168.10.0 0.0.0.255 area 0

Champagne_GatewayRouter(config-router)# network 80.0.1.0 0.0.0.7 area 0.0.0.2

Champagne_GatewayRouter(config-router)# default-information originate Champagne_GatewayRouter(config-router)# exit

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

TEST

Ping the tunnel interface on both routers, this should be successful, otherwise call the attention of your GA. On both Routers, issue the following show commands: (From privileged mode), type:

show crypto isakmp sa

show crypto ipsec sa

Now, we will configure Routing Configurations on the other Devices for a complete LAN-TO-LAN connectivity.

TASK 13: CONFIGURE  OSPF ON CISCO ASA

ASA-FW-IPS(config)# router ospf 1

ASA-FW-IPS(config-router)# network 172.16.32.0 255.255.255.248 area 0.0.0.1

ASA-FW-IPS(config-router)# network 10.144.1.0 255.255.255.0 area 0.0.0.1

ASA-FW-IPS(config-router)# exit

NOTE:

By default, the echo-reply ICMP is not allowed from a lower security to a higher security interface.

Enabling ICMP inspection in the policy-map global_policy:

- Firstly, check if class-map called “inspection-default” is preconfigured on the firewall:

ASA-FW-IPS(config)# sh run class-map

If yes, proceed to step 3 below, otherwise proceed to step 2 to create it.

- Create access-list and link it to the class map.

ASA-FW-IPS(config)# access-list 150 permit icmp any any

ASA-FW-IPS(config)# class-map inspection_default

ASA-FW-IPS(config-cmap)# match access-list 150

ASA-FW-IPS(config-cmap)# exit

- Create a policy map for icmp inspection

ASA-FW-IPS(config)# policy-map global_policy

ASA-FW-IPS(config-pmap)# class inspection_default

ASA-FW-IPS(config-pmap-c)# inspect icmp

ASA-FW-IPS(config-pmap-c)# exit

TASK 14:CONFIGURE THE INTERNAL ROUTER

Configure OSPF on the router

Internal_Router(config)# router ospf 1

Internal_Router(config-router)# network 10.234.100.0 0.0.0.255 area 0.0.0.1

Internal_Router(config-router)# network 172.16.32.0 0.0.0.7 area 0.0.0.1 Internal_Router(config-router)# router-id 172.16.32.1

Internal_Router(config-router)# exit

Internal_Router(config)# exit

TASK 15: CONFIGURE PALO ALTO FIREWALLS ON BOTH SITES

(A). For EIU Main Campus Site Palo Alto

Log on to the Palo Alto firewall and follow below steps:

- Click on Network tab.

- Click Virtual Routers on the left-hand pane.

- Click on default router configuration.

- On the Interfaces section displayed on the Virtual Router window, click Add.

- Check: Select all three ethernet interfaces i.e. ethernet1/1, ethernet1/2 and ethernet1/3 (click on each square at the left of each one).

- Then click on OSPF.

- On the OSPF Routing section, tick Enable and uncheck Reject Default Route.

- Enter 90.0.1.1 as Router ID

- Click Add under Areas tab.

- For Area ID type 0.0.0.1

- Click the Interface tab, then click ADD; then using the drop-down menu, select the interfaces ethernet1/1, ethernet1/2, and ethernet1/3 one after the other. Click OK each time to add the interfaces to the dynamic routing. Click OK when done.

- Click on Policies tab.

- Click on Security at the left-hand pane, and click Add.

- Under General tab, type ACCESS-POLICY as Name.

- Under Source tab, click Add under Source Zone, select Trust.

- Under Destination tab, click Add under Destination Zone, select Untrust.

- Ensure Action setting under Actions tab is set to Allow.

- Click OK.

INSTRUCTION: ** Please commit your configurations before proceeding.

(B). For the Champagne Remote Site Palo Alto

Log on to the Palo Alto firewall and follow below steps:

- Click on Network tab.

- Click Virtual Routers on the left-hand pane.

- Click on default router configuration.

- CHECK: On the Interfaces section displayed on the Virtual Router window, click Add.

- Select the ethernet interfaces one after the other i.e. ethernet1/1, ethernet1/2

- Then click on OSPF.

- On the OSPF Routing section, tick Enable and uncheck Reject Default Route.

- Enter 80.0.1.1 as Router ID.

- Click Add under Areas tab.

- For Area ID type 0.0.0.2

- Click the Interface tab, then click ADD; then using the drop-down menu, select the interfaces ethernet1/1, and ethernet1/2 one after the other. Click OK each time to add the interfaces to the dynamic routing. Click OK when done.

- Click on Policies tab.

- Click on Security at the left-hand pane, and click Add.

- Under General tab, enter ACCESS-POLICY as Name.

- Under Source tab, click Add under Source Zone, select Trust.

- Under Destination tab, click Add under Destination Zone, select Untrust.

- Ensure Action setting under Action tab is set to Allow.

- Click OK.

INSTRUCTION: ** Please commit your configurations before proceeding.

TEST

On the Internal Router, EIU Main Router and Champagne Router, From the privileged mode, issue the commands below:

Show ip route ospf – Can you see any OSPF routes in the routing table?

TASK 16: NAT AND ACCESSING THE INTERNET

The purpose of this task is to configure Network Address Translation (NAT) necessary for internet accessibility. NAT can allow multiple private IP addresses to access the internet with a single shared public IP address. We will also be introduced to static NAT, by statically NATing the private IP address of the web server to our public IP address to enable online access to the webserver from the internet.

*******************************************************************************

Before you configure NAT, test if you can access the webserver:

From the two sites, open the web-browser of any of the host and type: http://172.16.1.2

This should not be successful, I bet you know why!!

*******************************************************************************

Solution Guide

- On the EIU Main Campus Gateway Router, Create Access-list for the permitted traffic.

Campus-Gateway> enable

Campus-Gateway# conf t

Campus-Gateway(config)# access-list 105 permit ip any any

The access-list is permitting all traffic from the internal network to any network on the internet.

- Configure a default route to the ISP

Campus-Gateway(config)# ip route 0.0.0.0 0.0.0.0 172.29.129.254

- Configure NAT

Campus-Gateway(config-if)# int g0/0

Campus-Gateway(config-if)# ip nat outside

Campus-Gateway(config-if)# exit

Campus-Gateway(config)# int g0/1

Campus-Gateway(config-if)# ip nat inside

Campus-Gateway(config-if)# exit

Campus-Gateway(config)# ip nat inside source list 105 interface g0/0 overload

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

NAT Overloading or Port Address Translation (PAT) is a modified form of dynamic NAT where the number of inside local addresses is greater than the number of inside global addresses. Mostly, there is just a single inside global IP address providing Internet access to all inside hosts.

Configure Static NAT for internet users to be able to access the web server using EIU Main Gateway router public facing interface IP address.

- On the Champagne Remote site Router. Create Access-list for the permitted traffic.

Champagne_GatewayRouter> enable

Champagne_GatewayRouter# conf t

Champagne_GatewayRouter(config)# access-list 105 permit ip any any

NOTE: The access-list is permitting all traffic from the internal network to any network on the internet.

- Configure a default route to the ISP.

Champagne_GatewayRouter(config)# ip route 0.0.0.0 0.0.0.0 172.29.129.254

- Configure NAT

Champagne_GatewayRouter(config-if)# int g0/0

Champagne_GatewayRouter(config-if)# ip nat outside Champagne_GatewayRouter(config-if)# exit

Champagne_GatewayRouter (config)# int g0/1

Champagne_GatewayRouter(config-if)# ip nat inside Champagne_GatewayRouter(config-if)# exit

Champagne_GatewayRouter (config)# ip nat inside source list 105 interface g0/0 overload

INSTRUCTION: Use “show run” to re-check the parameters inputted in this task.

TEST

Test to see that ALL LAN Hosts can browse to internet now. If Unsuccessful, call the attention of your GA.

At this stage, no one can connect to the web server. To make this happen, follow below guides on Main Campus Router and Palo alto.

Fig. 13

- For the EIU Main Campus:

First, check what is the IP address assigned through DHCP to the Campus-Gateway G0/0 interface.

Campus-Gateway# show ip interface brief

The IP address should be in the 172.29.129.0/24 network (Provided by DHCP through the Internet).

After verifying, type the following command:

Campus-Gateway(config)# ip nat inside source static tcp 90.0.1.1 80 172.29.129.x 80 extendable

INSTRUCTION: Replace the x in “172.29.129.x” for the corresponding number assigned through DHCP by the internet.

Here the router is configured to NAT to the Palo Alto Outside interface (90.0.1.1) which is then configured to do static NATing to the web server which is 172.16.1.2.

Configure the static NAT on Palo Alto to the web server (Only on the EIU Main Campus Network).

Fig. 14

- On the Palo Alto at Main Campus, follow the steps below to configure NAT.

- Click the Policies tab, Choose NAT On the left pane.

- Click Add, under General tab, type WEBSERVER-NAT.

- NAT Type should be set to IPV4.

- Click Original Packet tab.

- Under Source Zone, click Add, then select DMZ.

- Under Destination Zone, use the drop-down menu to select Untrust, for Destination Interface select ethernet1/1, for Service, select service-http.

- Under Source Address, click Add, click Address (in blue), on the new dialogue box, enter WebServer. Type 172.16.1.2/32 in space provided in front of IP Netmask and click OK.

- Click on Translated Packet, under Source Address Translation, select Static IP as Translation Type.

- Type 90.0.1.1/32 as Translated Address.

- Click on Yes for Bi-directional.

- Click OK.

- Configure the Security Policy to allow both external and Internal access to the web server

- Under the Policies tab.

- Click on Security on the left-hand pane.

- Click Add.

- Under General tab, type DMZ-Policy as Name

- Click Source tab and click Add.

- Under Source Zone, click Add, and select both Trust and Untrust Zones.

- Click on Destination tab, under Destination Zone, click Add, select DMZ.

- Click Service/URL Category, under Service click Add and select service-http.

- Under Actions tab, ensure Allow is selected.

- Click OK.

- Click Commit at the top right corner of the Palo Alto and follow the prompts..

Fig. 15

(If you do not see exactly what is depicted in figure 15, please call the attention of your instructor)

TASK 17: ACCESSING THE WEBSERVER

Access the Web Server:

- Press Ctrl + Alt + Shift on the keyboard.

- Change the Input Method to On-Screen Keyboard.

- Using the On-Screen Keyboard, press Ctrl + Alt + Del.

- Press Ctrl + Alt + Shift again to change the Input Method back to none.

- When prompted for the password, type Test123 to access.

Setup the Web Server

INSTRUCTION: Configure the IP address of the web server as 172.16.1.2/24. Configure the default gateway as 172.16.1.1 (Palo Alto eth1/3 interface).

Enable IIS and the required IIS components:

- Open Server Manager and click Manage > Add Roles and Features. Click Next.

- Select Role-based or feature-based installation and click Next.

- Select the appropriate server. The local server is selected by default. Click Next.

- Enable Web Server (IIS) and click Next.

- No additional features are necessary to install the Web Adaptor, so click Next.

- On the Web Server Role (IIS) dialog box, click Next.

- On the Select role services dialog box, leave everything as default and click Next.

- Verify that your settings are correct and click Install.

- When the installation completes, click Close to exit the wizard.

Add a website:

- Create a folder named “Web” on Local Disk (C:)

- On the Server Manager Dashboard, click Tools > Internet Information Services (IIS) Manager.

- In the Connections pane, click to expand the server entry, then right-click Sites.

- Click Add Website...

- Fill out the following information:

- The website will be added, and you will be able to view the website's files in a browser.

FINAL TEST

- Keeping only the devices on the EIU Main Campus on, test the connection of the LAN hosts to the internet. You can do this by pinging any known domain name like google.com, yahoo.com or by launching a web browser on the Windows PC on LAN and visiting any registered website.

Is this successful?

Ans: Yes.

- Keeping only the devices on the EIU Main Campus on, test the connection of the LAN hosts to the Webserver. Open your web browser on the Windows PC and type: http://172.16.1.2 and press enter.

Is this successful?

Ans: Yes

- Keeping only the devices on the Champagne Satellite Campus on, test the connection of the LAN host to the internet. You can do this by pinging any known domain name like google.com, yahoo.com (from the command prompt) or by launching a web browser on the Windows PC on LAN and visiting any registered website.

Is this successful?

Ans: Yes

- Keeping only the Legitimate Remote Access User and the devices on the EIU Main Campus on except the Windows PC on the main campus LAN, test the connection of the Legitimate RAU to the internet.

Is this successful?

Ans: Yes

- Keeping only the Legitimate Remote Access User and the devices on the EIU Main Campus on except the Windows PC on the main campus LAN, test the connection of the Legitimate RAU to the Windows server (type http://172.16.1.2) on the web browser.

Is this successful?

Ans: No, 172.16.1.2 is not known on the internet. Rather 172.29.129.3 is known and works.

- Keeping the devices on the Main Campus and the Satellite Campus on except the Windows PC on the Main Campus, test the connection between the LAN host on the Satellite Campus and the Webserver. Type http://172.16.1.2 on the web browser.

Is this successful?
Ans: Yes

Fig 3: reproduced for your convenience.